# Inflatable Assault Course - Risk Assessment

| Company: | King of the castles Entertainments | Location (Site / Building / Room): | Events |
| --- | --- | --- | --- |
| Assessment Date: | 15 March 2026 | Review Date: | 31 March 2027 (or if changes to the risk) |
| Assessor’s Name: | Natalie king | Job Title: | DIRECTOR |
| Description of Task: | Transportation, Assembly and Use of Inflatable Assault Course Equipment |  |  |

| Description of the hazard (or hazardous event) | Who might be at risk? | What is the risk? | Evaluation and Control Measures | Current risk rating* |  |  | Identify any additional controls that may be needed | Timescale for additional controls and responsible person | Risk Rating |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | C | R |  |  | L (Low) | M (Medium) | H (High) |
| Loading, Handling and Transportation of Equipment | Operator / Spectator | Injury whilst handling equipment | - Equipment must be handled in the correct manner. - Manual handling should be reduced to a minimum by using available trolleys / trucks. - Ramps / Lifts to be used when required and available. - Loads must be correctly secured. - Individuals are not to undertake anything that puts themselves or anyone at risk. | 1 | 2 | 2 | - Where required individuals can assist in handling the equipment and adhering to site Manual Handling measures. - Staff are to wear PPE at all times in accordance with the Clients requirements and site guidelines. | N/A | 2 | N/A | N/A |
| Installation of Equipment on site | Operator / Spectator Participants | Injury whilst installing the equipment Damage to venue | a). The equipment is inspected before installation to ensure it is free from damage. | 1 | 2 | 2 | 1.The equipment is secured with 30cm long 20mm diameter metal pegs if installed on grass. | N/A | 2 | N/A | N/A |

| Installation of Equipment on site (Cont.) | Operator / Spectator Participants | Injury whilst installing the equipment Damage to venue | - The Operators must have clear access to the setup area. - The setup area must be level and free from any debris. - The operator must be notified of any underground services, cable, weight limits, or external factors that may hinder installation of this equipment. - The equipment should not be installed near overhead power lines. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ground Type | Operator / Spectator Participants | Injury to Participant | - The equipment can be installed on grass. - The area must be free from debris and animal waste. | 1 | 1 | 1 | N/A | N/A | 1 | N/A | N/A |
| Electricity and Cabling | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants | - Equipment requires access to 13Amp plug socket. - Power source should be no further than 40m away from equipment. - All cabling to be covered where required and routed away from Walkways etc. | 1 | 2 | 2 | 1.Cable matting supplied and used where any cable crosses a walkway. | N/A | 2 | N/A | N/A |

| Electricity and Cabling (cont.) | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants | d). All electrical components are inspected for wear and that they are covered by the 6 month PAT test. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation of Equipment on site | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants | - Users will be instructed to form a queue in front of the equipment in view of the operator who will control the queue. - Users must follow the operator’s instructions at all times. - Users must go through the equipment in the same direction no users are allowed to go against the flow. - Safety Mats must be placed at the entry and exit points of the inflatable. | 1 | 1 | 1 | - Spectators are to be a minimum of 2m distance away from the equipment and not allowed to sit / rest or interfere with the equipment at any time. - Where applicable, equipment is to be cleaned after use and using approved cleaning materials. | N/A | 1 | N/A | N/A |
| User Safety | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants | - Participants who appear to be under the influence of alcohol / drugs will be refused access to the equipment. - The Operator reserves the right to refuse permission to anyone they feel may be unsafe to partake in the activity. | 1 | 1 | 1 | - Where applicable, a Trained Operator will be in control of the equipment at all times who is briefed on the safe running of the equipment. - Trained Operators receive annual training and updates on the equipment operation. - The Fun Experts hold no liability for the onset of any medical condition or injury | N/A | 1 | N/A | N/A |

| User Safety (cont.) | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants | - Participants must be wearing suitable clothing for the activity. - Items of loose clothing must be secured and any watches, jewellery, glasses must be removed or secured prior to taking part in the activity. - Children must be supervised by an adult at all times. - Food and Drink inclusive of chewing gum and sweets will not be allowed in the operating area. - Participants should be aware of their own physical ability and notify the Operator if they want to end the activity. - Anyone suffering from Heart Disease, physical mobility restrictions, including broken bones, and those who are pregnant should not be allowed to take part in this activity. - The equipment must be used in accordance with the Manufacturers guidelines. |  |  |  | sustained from partaking in this activity. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Weather | Operator / Spectator Participants | Injury to Operator Injury to Spectator Injury to Participants (slip / trip/ falls) | a). The equipment can be used indoors and outdoors in mild weather conditions. - The wind speed should be monitored during the course of the event by the operator and hirer and should not exceed the maximum permitted wind speed of 19mph. - Should winds exceed 19mph the equipment must be cleared of all participants and deflated until it is safe to inflate and resume the activity. - In the event of rain, the operator or hirer on the day will assess the conditions and equipment to ensure safe operation. - The equipment must be dry and safe before resuming the operation. | 1 | 1 | 1 | 1.Winds Speeds are monitored / measured by a supplied Anemometer. | N/A | 1 | N/A | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Risk Rating Calculator	(Likelihood x Consequence = Risk)

| Likelihood that hazardous event will occur |  | Consequence of hazardous event |  |
| --- | --- | --- | --- |
| 1 | Very unlikely | 1 | Insignificant (no injury) |
| 2 | Unlikely | 2 | Minor (minor injury requiring first aid only) |
| 3 | Fairly likely | 3 | Moderate (Up to three days absence) |
| 4 | Likely | 4 | Major (More than seven days absence) |
| 5 | Very likely | 5 | Catastrophic (Permanent injury or death) |

Action Level Table

| Risk Rating | Risk Level | Actions to be taken |
| --- | --- | --- |
| 20 – 25 | Very High Risk | STOP! Stop the activity and take immediate action to reduce the risk, a detailed plan should be developed and implemented before work commences or continues. Senior management should monitor the plan. |
| 15 – 16 | High Risk | Urgent Action! Take immediate action and stop the activity if necessary, maintain existing controls rigorously. The continued effectiveness of control measures should be monitored periodically. |
| 8 – 12 | Moderate Risk | Action Moderate risks may be tolerated for short periods only while further control measures to reduce the risk are being planned and implemented. Improvements should be made within specified timescales. |
| 3 – 6 | Low Risk | Monitor Look to improve at the next review or if there is a significant change. Monitor the situation periodically to determine if new control measures are required. |
| 1 – 2 | Very Low Risk | No Action No further action is usually required, but ensure that existing controls are maintained and reviewed regularly. |